Task: CanSat Flight-Event Sequencer (C99, no heap)

Context

A CanSat (STM32F407, Cortex-M4F) is carried by a balloon to about 1500 m and released. It descends under a parachute. During descent, a payload section holding a biological sample must be separated from the carrier by a servo at 100 m above the launch pad. There is no telecommand: the decision is fully autonomous. Telemetry is sent to the ground station once per second.

Implement flight_step() declared in flight.h. It is called every 10 ms with one Sample. Implement flight_init() too.

Definitions

- A sample is valid if press_valid != 0 and 30000 <= press_pa <= 110000. When press_valid == 0 the value of press_pa is undefined (the driver may leave a default there).

- AGL = 44330 * (1 - (press_pa / P0)^0.19029) in metres, where P0 is the pad baseline pressure (below). Use double precision.

- A valid sample is accepted if |AGL - last_accepted_AGL| <= 50 m, or if it is the 100th (or later) consecutive valid-but-rejected sample (re-sync). Initial last_accepted_AGL = 0.

- Samples that are invalid or rejected are bad. Bad samples change no phase logic and do not touch debounce counters. After 30 consecutive bad samples, all debounce counters are reset to 0 (once).

- All counters count accepted samples only. A counter resets to 0 when an accepted sample fails its condition.

Phases (Outputs.phase)

| Phase | Rule to leave it |
| --- | --- |
| PAD (0) | AGL > 30 m for 20 consecutive accepted samples -> ASCENT |
| ASCENT (1) | Track max_AGL. When max_AGL >= 500 m and AGL <= max_AGL - 100 m for 30 consecutive accepted samples -> DESCENT |
| DESCENT (2) | AGL <= 100 m for 25 consecutive accepted samples -> SEPARATED and the servo command is issued |
| SEPARATED (3) | AGL <= 15 m and acceleration magnitude between 800 and 1200 mg for 100 consecutive accepted samples -> LANDED |
| LANDED (4) | terminal |

Pad baseline: P0 starts as the first valid pressure. While in PAD, for every accepted sample with |press_pa - P0| <= 30, update P0 += (press_pa - P0) / 64 using C integer division (truncation toward zero). P0 is frozen when PAD is left. Compute AGL with the P0 value from before the update.

Outputs

- servo_us: 1000 until separation, 2000 from the separation sample onwards. It must change exactly once and never go back.

- buzzer: 1 if and only if the phase is LANDED.

- Telemetry: on the first call, and then whenever (uint32_t)(t_ms - last_tx_ms) >= 1000, set tele_ready = 1 and write the ASCII packet

`SHD,<seq>,<trel_s>.<trel_ms3>,<agl_m>,<press_pa>,<ax>,<ay>,<az>,<phase>\r`

  - seq starts at 0 and increments per packet.

  - trel = time since DESCENT was entered (0 before DESCENT), computed with unsigned 32-bit subtraction so it survives the t_ms wrap. Print seconds and a 3-digit millisecond field.

  - agl_m = last accepted AGL rounded with floor(x + 0.5).

  - press_pa, ax, ay, az are the raw values of the current sample.

  - tele_len excludes the terminating NUL. The buffer is 80 bytes.

Constraints

- C99, no heap (malloc/calloc/realloc/free must not appear in the object file), no recursion, no blocking calls.

- FlightState has a fixed 256-byte size. You may cast it to your own private struct.

- flight_step must be reentrant-free of globals except the state passed in.

Deliverables

- flight.c implementing the functions.

- Your solution must pass python3 run_tests.py flight.c.